Lab Exercise 1- Structure of a Modelica Model & Acausal vs Causal Modeling

1. Aim

To understand:

- Structure of a Modelica model

  - Parameters

  - Variables

  - Equations

- Difference between acausal and causal modeling

- How equation-based modeling works in OpenModelica

2. Learning Outcomes

After completing this lab, students will be able to:

- Create a Modelica model

- Differentiate parameters and variables

- Write physical equations

- Understand acausal modeling

- Compare causal vs acausal approach

3. Software Required

- OpenModelica (OMEdit)

4. PART A – Structure of a Modelica Model

We will create a Mass-Spring-Damper System, commonly used in aerospace and defense vibration systems.

STEP 1: Open OpenModelica

- Launch OMEdit

- Click File → New Model

- Name the model: MassSpringSystem

Click OK.

STEP 2: Understand Basic Model Structure

Basic Modelica structure:

model ModelName

// Parameters

parameter Real p;

// Variables

Real x;

equation

// Equations

x = p;

end ModelName;

Modelica has 3 main parts:

- Parameters

- Variables

- Equations

STEP 3: Add Parameters

Parameters are constants defined before simulation.

Add:

parameter Real m = 10;     // mass (kg)

parameter Real k = 200;    // spring constant (N/m)

parameter Real c = 20;     // damping coefficient

Explanation:

- m → mass of object

- k → spring stiffness

- c → damping factor

These values remain constant during simulation.

STEP 4: Add Variables

Variables change during simulation.

Add:

Real x(start=0.1);     // displacement

Real v(start=0);       // velocity

Explanation:

- x → position

- v → velocity

- start value helps initialize simulation

STEP 5: Add Equations

Add physics equations:

equation

der(x) = v;

m*der(v) + c*v + k*x = 0;

Explanation:

- der(x) = v → velocity is derivative of position

- mder(v) + cv + k*x = 0 → Newton’s law

This describes vibration motion.

COMPLETE CODE

model MassSpringSystem

// Parameters

parameter Real m = 10;

parameter Real k = 200;

parameter Real c = 20;

// Variables

Real x(start=0.1);

Real v(start=0);

equation

der(x) = v;

m*der(v) + c*v + k*x = 0;

end MassSpringSystem;

STEP 6: Simulate

- Click Check Model

- Click Simulate

- Plot:

  - x

  - v

You will observe oscillation with damping.

PART B – Understanding Each Section in Detail

1. Parameters

Defined using:

parameter Real name = value;

Properties:

- Constant during simulation

- Used for physical constants

- Improves model flexibility

If you change m from 10 to 20, behavior changes.

2. Variables

Defined using:

Real variableName;

Properties:

- Solved by equation solver

- Change over time

- Represent system states

3. Equations

Defined under:

equation

Important:

This is NOT assignment like C or MATLAB.

It is a mathematical relationship.

Example:

m*der(v) + c*v + k*x = 0;

Means:

All variables must satisfy this equation simultaneously.

PART C – Acausal vs Causal Modeling

STEP 7: Understand Causal Modeling

Causal modeling defines direction.

Example in algorithm style:

acceleration = (-c*v - k*x)/m;

v = v + acceleration*dt;

x = x + v*dt;

Here:

- Input/output defined

- Order matters

- Step-by-step execution

This is used in MATLAB or C.

STEP 8: Acausal Modeling (Modelica)

Modelica version:

m*der(v) + c*v + k*x = 0;

No input/output specified.

The solver:

- Determines unknowns

- Solves equations simultaneously

You define physics, not computation order.

KEY DIFFERENCE

Causal → Algorithm
Acausal → Physical law

PART D – Defense-Based Example

Consider a Tank Suspension System.

Physics:

mder(v) + cv + k*x = TerrainForce

You write:

model TankSuspension

parameter Real m = 20000;

parameter Real k = 500000;

parameter Real c = 5000;

Real x(start=0.2);

Real v(start=0);

Real terrainForce;

equation

terrainForce = 10000*sin(time);

der(x) = v;

m*der(v) + c*v + k*x = terrainForce;

end TankSuspension;

Simulation shows vibration under rough terrain.

Defense Use:

- Improve shock absorption

- Increase crew safety

- Optimize suspension design