Lab Exercise 3- Navigating OMEdit & Drag-and-Drop Modeling Mechanical and Electrical Systems

PART A

MECHANICAL MODEL – MASS SPRING DAMPER WITH SINUSOIDAL FORCE

STEP 1: Open OMEdit

Launch OpenModelica → Open OMEdit.

STEP 2: Create a New Model

Click File → New Model

Enter model name: MassSpringElectricalLab

Click OK

STEP 3: Add Mechanical Components

From Library Browser:

Modelica → Mechanics → Translational → Components

Drag the following components into workspace:

• Mass
• Spring
• Damper
• Fixed

Arrange them in order:

Fixed → Spring → Damper → Mass

STEP 4: Connect Mechanical Components

Click the Connect Tool

Make connections:

Fixed.flange → Spring.flange_a
Spring.flange_b → Damper.flange_a
Damper.flange_b → Mass.flange_a

STEP 5: Add Force Source

From:

Modelica → Mechanics → Translational → Sources

Drag:

• Force

Connect:

Force.flange → Mass.flange_b

STEP 6: Add Sine Signal Block

From:

Modelica → Blocks → Sources

Drag:

• Sine

STEP 7: Configure Sine Block

Double-click Sine

Set:

amplitude = 100
freqHz = 1

Click OK

STEP 8: Connect Signal to Force

Connect:

Sine.y → Force.f

This supplies:

f(t) = 100 sin(2πt)

STEP 9: Set Mechanical Parameters

Double-click components and set:

Mass:

m = 10

Spring:

c = 200

Damper:

d = 20

STEP 10: Check and Simulate

Click:

Check Model

Then:

Simulate

Simulation time:

Start = 0
Stop = 10

STEP 11: Plot Results

Plot:

Mass.s
Mass.v
Force.f

Observe damped oscillation.

PART B

ELECTRICAL CIRCUIT DRAG-AND-DROP MODEL

We now create a simple RLC circuit using Modelica Standard Library.

Electrical System Description

Series RLC circuit excited by sinusoidal voltage.

Applications:

• Radar power circuits
• Communication electronics
• UAV onboard electronics
• Defense sensor systems

STEP 12: Create a New Model (or New Diagram)

Click:

File → New Model

Name: ElectricalCircuitModel

Click OK.

STEP 13: Add Electrical Components

From:

Modelica → Electrical → Analog → Basic

Drag:

• Resistor
• Inductor
• Capacitor
• Ground

From:

Modelica → Electrical → Analog → Sources

Drag:

• SineVoltage

STEP 14: Arrange Components

Place in this order:

SineVoltage → Resistor → Inductor → Capacitor → Ground

STEP 15: Connect Electrical Components

Make connections:

SineVoltage.p → Resistor.p
Resistor.n → Inductor.p
Inductor.n → Capacitor.p
Capacitor.n → Ground.p
SineVoltage.n → Ground.p

Make sure every circuit has Ground connected.

STEP 16: Set Electrical Parameters

Double-click and set:

Resistor:

R = 100

Inductor:

L = 0.5

Capacitor:

C = 0.01

SineVoltage:

V = 10

freqHz = 1

STEP 17: Check and Simulate

Click:

Check Model

Then:

Simulate

Set simulation:

Start = 0
Stop = 5

STEP 18: Plot Results

Plot:

Capacitor.v
Inductor.i
SineVoltage.v

Observe charging and oscillatory behavior.

Understanding What Happened

Mechanical System Equation (Automatically Formed):

m * der(v) + dv + kx = 100 sin(2πt)

Electrical System Equation:

L di/dt + R i + (1/C) ∫i dt = V sin(2πt)

You did NOT manually write these equations.

Modelica generated them from connections.

Concept Comparison

Mechanical domain → Force balance
Electrical domain → Kirchhoff laws

Both use:

Acausal connectors
Equation-based modeling
Automatic equation assembly