Lab Exercise 4 - Simple DC Circuit Using Electrical Library (Text Mode and Graphical View)

1. AIM

To model and simulate a simple DC circuit (Voltage source + Resistor) using Modelica Electrical library in text mode and graphical mode.

2. OBJECTIVE 1

Simple DC Circuit Using Graphical View in OMEdit

1. AIM

To design and simulate a simple DC circuit (Voltage Source + Resistor + Ground) using the graphical modeling interface of OMEdit.

2. OBJECTIVE

After completing this experiment, students will be able to:

• Navigate OMEdit graphical environment
• Drag components from Modelica Standard Library
• Connect electrical components
• Simulate and analyze results

3. PROBLEM STATEMENT

Create the following circuit:

10V Battery → 5Ω Resistor → Ground

Verify:

• Current through resistor
• Voltage across resistor

Expected result:

I = V / R = 10 / 5 = 2 A

4. STEP-BY-STEP PROCEDURE

STEP 1 – CREATE NEW MODEL

- Open OMEdit

- Click File → New Modelica Class

- Enter:

Name: SimpleDCCircuitGraphical

Restriction: model

- Click OK

- Stay in Diagram View

STEP 2 – OPEN MODELICA STANDARD LIBRARY

On the left panel:

Expand:

Modelica
→ Electrical
→ Analog

STEP 3 – ADD COMPONENTS (DRAG & DROP)

3.1 Add Voltage Source

Navigate to:

Modelica
→ Electrical
→ Analog
→ Sources
→ ConstantVoltage

Drag it into the diagram.

Double-click it and set:

V = 10

Click OK.

3.2 Add Resistor

Navigate to:

Modelica
→ Electrical
→ Analog
→ Basic
→ Resistor

Drag into diagram.

Double-click it and set:

R = 5

Click OK.

3.3 Add Ground

Navigate to:

Modelica
→ Electrical
→ Analog
→ Basic
→ Ground

Drag into diagram.

STEP 4 – CONNECT COMPONENTS

Now click the Connect tool (wire icon).

Connect as follows:

Voltage positive terminal (p) → Resistor positive terminal (p)

Resistor negative terminal (n) → Ground

Voltage negative terminal (n) → Ground

Your final diagram should look like:

Battery → Resistor → Ground
And battery negative connected to ground.

5. WHAT DOES THIS CIRCUIT REPRESENT?

This is a simple DC circuit:

Battery provides 10 volts.
Resistor limits current to 2 amps.
Ground provides reference (0V).

6. WHAT EQUATIONS DOES MODELICA AUTOMATICALLY GENERATE?

You did NOT write equations manually.

OMEdit automatically generates:

Voltage equation:
V.p.v − V.n.v = 10

Resistor equation:
R.p.v − R.n.v = 5 × R.p.i

Kirchhoff Current Law at connection nodes.

The system solves:

I = 10 / 5 = 2 A

7. STEP 5 – SIMULATE

- Click Simulate button

- Use default settings:

  - Start Time = 0

  - Stop Time = 1

Click Simulate.

8. STEP 6 – PLOT RESULTS

After simulation:

Plot:

R.p.i → Current through resistor
R.p.v → Voltage at resistor

Expected:

Current = 2 A (constant)
Voltage across resistor = 10 V

2. OBJECTIVE 2

Simple DC Circuit Using Text View in OMEdit

• Use Electrical library in text editor
• Define components using code
• Connect components using connect()
• Simulate and observe current and voltage

3. PROBLEM STATEMENT

We will create a simple circuit:

Battery (10V) → Resistor (5 Ohm) → Ground

We will calculate:

• Current through resistor
• Voltage across resistor

4. THEORY

Ohm’s Law:

V = I × R

So,

I = V / R

If:

V = 10 V
R = 5 Ohm

Then:

I = 10 / 5 = 2 A

We expect current = 2 Amps.

5. STEP 1 – CREATE NEW MODEL

Open OMEdit.

File → New Modelica Class

Name: SimpleDCCircuit

Restriction: model

Click OK.

Switch to Text View.

6. COMPLETE CODE (WRITE THIS EXACTLY)

model SimpleDCCircuit

// Voltage source (10 Volt)

Modelica.Electrical.Analog.Sources.ConstantVoltage V(V=10);

// Resistor (5 Ohm)

Modelica.Electrical.Analog.Basic.Resistor R(R=5);

// Ground

Modelica.Electrical.Analog.Basic.Ground G;

equation

// Connect positive terminal of voltage to resistor

connect(V.p, R.p);

// Connect resistor to ground

connect(R.n, G.p);

// Connect negative terminal of voltage to ground

connect(V.n, G.p);

end SimpleDCCircuit;

7. EXPLAINING EACH PART

model SimpleDCCircuit

This defines a new Modelica model.

Voltage Source

Modelica.Electrical.Analog.Sources.ConstantVoltage V(V=10);

This creates a 10 Volt battery.

• V.p → positive terminal
• V.n → negative terminal

Resistor

Modelica.Electrical.Analog.Basic.Resistor R(R=5);

This creates a 5 Ohm resistor.

• R.p → positive side
• R.n → negative side

Ground

Modelica.Electrical.Analog.Basic.Ground G;

Every electrical circuit needs a reference (0 Volt).

connect() Statements

Modelica uses acausal modeling.

We do not write equations like:

I = V/R

Instead, we connect components physically.

Example:

connect(V.p, R.p);

Means: connect battery positive to resistor positive.

8. WHAT EQUATIONS DOES MODELICA AUTOMATICALLY GENERATE?

From voltage source:

V.p.v - V.n.v = 10

From resistor:

R.p.v - R.n.v = R × R.p.i

Kirchhoff Current Law is automatically applied at connection nodes.

Modelica solves:

V = IR

Internally.

9. SIMULATION

Right-click model → Simulate.

Use default settings:

Start Time = 0
Stop Time = 1

After simulation, plot:

R.p.i
R.p.v

10. EXPECTED RESULT

Current:

2 A (constant)

Voltage across resistor:

10 V

Because:

I = 10 / 5 = 2 A