ASSIGNMENT 4- Converting Mechanical Graphical Model into Text Mode

1. AIM

To recreate a given mechanical model (created in Graphical View) using Text View in OMEdit by manually writing Modelica code.

2. OBJECTIVE

After completing this assignment, students will be able to:

• Declare Modelica mechanical components in Text View
• Assign correct parameter values
• Use connect() statements properly
• Understand the relationship between graphical modeling and text-based modeling
• Simulate and interpret mechanical system response

3. PROBLEM DESCRIPTION

You are given a mechanical system designed in Graphical Mode with the following structure:

Constant Force → Mass → Spring → Damper → Fixed Support

The parameter values used in the graphical model are:

Mass (m) = 2 kg
Spring stiffness (c) = 800 N/m
Damping coefficient (d) = 40 Ns/m
Applied force (f) = 100 N

Your task is to recreate this exact system using Text View only.

4. TASKS TO BE PERFORMED

STEP 1 – Create New Model

Create a new model named:

MechanicalSystemText

Switch to Text View before writing any code.

No graphical components should be used for this task.

STEP 2 – Declare All Components

Inside the model, declare the following components:

A constant force source named forceInput
A mass component named massBlock
A spring component named springElement
A damper component named damperElement
A fixed support named fixedSupport

All components must use the same parameter values as provided in the problem description.

Ensure:

• Proper component naming
• Correct parameter assignment
• No missing declarations

STEP 3 – Write Correct connect() Statements

Inside the equation section, write connect() statements that replicate the physical structure exactly as in the graphical model:

forceInput → massBlock
massBlock → springElement
springElement → damperElement
damperElement → fixedSupport

Ensure:

• Proper flange-to-flange connections
• Correct physical order
• No incorrect or missing connections

The connection order must represent the same mechanical chain.

STEP 4 – Complete Text Mode Code

Your final model must include:

• model declaration
• Component declarations
• Parameter assignments
• equation section
• All connect() statements
• Proper end statement

The model must compile without errors.

5. SIMULATION REQUIREMENTS

After completing the model:

Simulate using:

Start Time = 0
Stop Time = 10

Variables to Plot

Plot the following:

• massBlock.s (Displacement)
• massBlock.v (Velocity)