# Modelica & OpenModelica in Defense Systems

# 1. Modelica for Defense Systems

# Defense systems are:

- Multi-domain (mechanical + electrical + thermal + control)

- Highly dynamic

- Safety-critical

- Performance-constrained

- Interdisciplinary

# Modelica is particularly suitable because:

- It supports multi-physics modeling

- It uses acausal equations (natural physical laws)

- It allows system-level integration

- It supports dynamic simulation over time

# OpenModelica enables:

- Cost-effective research and development

- Academic defense research labs

- Rapid prototyping and validation

# Use Case 1: Missile Flight Dynamics Modeling

# System: Surface-to-Air Missile

# Problem

# Analyze:

- Acceleration profile

- Thrust-to-weight ratio

- Aerodynamic drag

- Trajectory prediction

- Fuel burn rate

# Modelica Representation

# Newton’s law:

# m * der(v) = Thrust − Drag − m*g

# Height dynamics:

# der(h) = v

# Fuel consumption:

# der(m) = −fuelBurnRate

# What OpenModelica Provides

- Time-based trajectory simulation

- Velocity vs altitude curves

- Stability validation

- Failure case simulation

# Defense Value

- Improve guidance accuracy

- Validate propulsion system

- Optimize fuel efficiency

- Study worst-case scenarios

# Use Case 2: Radar System Power & Thermal Modeling

# System: Mobile Battlefield Radar Unit

# Problem

# Radar generates:

- High electrical load

- Thermal heat buildup

- Cooling demand

- Battery drain

# Modelica Capabilities

# Electrical domain:

# V = R * I

# Thermal domain:

# Q = m * Cp * dT

# Cooling dynamics:

# HeatGenerated − HeatRemoved = mCpder(T)

# OpenModelica Simulation

- Thermal rise over time

- Cooling system efficiency

- Power distribution optimization

- Emergency shutdown validation

# Defense Importance

- Prevent overheating

- Increase radar uptime

- Improve energy efficiency

- Enhance battlefield reliability

# Use Case 3: Armored Vehicle Suspension System

# System: Main Battle Tank Suspension

# Problem

# Analyze:

- Shock absorption

- Terrain impact

- Ride stability

- Weapon stabilization

# Modelica Modeling

# Mass-spring-damper equation:

# mder(v) + cv + k*x = TerrainForce

# OpenModelica Simulation

- Off-road impact response

- Vibration frequency analysis

- Suspension parameter optimization

- Stability under extreme terrain

# Defense Outcome

- Improved crew safety

- Better weapon accuracy

- Reduced mechanical wear

- Enhanced mobility

# Use Case 4: Integrated Power Management in Naval Systems

# System: Naval Combat Ship Power Grid

# Problem

# Multiple subsystems:

- Radar

- Missile launch systems

- Communication arrays

- Propulsion system

# Need:

- Stable power distribution

- Emergency load shedding

- Redundancy validation

# Modelica Capabilities

# Multi-domain coupling:

# Electrical + mechanical + control

# Load balance equation:

# TotalGeneration = TotalLoad

# Dynamic stability:

# Lder(I) + RI = V

# OpenModelica Simulation

- Power fluctuation scenarios

- Failure mode analysis

- Load redistribution

- Emergency response simulation

# Defense Significance

- Prevent blackout during combat

- Validate redundancy architecture

- Improve mission reliability

# Use Case 5: Autonomous Ground Vehicle (Military Rover)

# System: Autonomous Recon Vehicle

# Model Includes

- Drive train dynamics

- Battery discharge

- Motor control

- Obstacle detection response

- Control algorithms

# Modelica allows:

- Multi-domain integration

- Mechanical + electrical + control in one model

# OpenModelica simulates:

- Terrain slope impact

- Energy efficiency

- Emergency braking

- Autonomous navigation stability

# 8. Why OpenModelica is Strong for Defense Research

- Open-source and customizable

- Supports FMI (co-simulation)

- Handles large-scale dynamic systems

- Enables rapid prototyping

- Reduces dependency on expensive proprietary tools