# Basics of Modelica Language

# 1. Introduction to Modelica Language

# Modelica is an equation-based, object-oriented modeling language used for simulation of multi-domain physical systems. It allows engineers to model mechanical, electrical, thermal, hydraulic, and control systems in an integrated manner.

# Unlike procedural programming languages, Modelica focuses on describing physical relationships using equations rather than writing step-by-step algorithms.

# 2. Structure of a Modelica Model

# A Modelica model typically consists of:

- Parameters

- Variables

- Equations

- Components (optional for complex systems)

# The basic structure looks like:

# model ModelName

# parameter Real p1;

# Real x;

# Real y;

# equation

# equation_1;

# equation_2;

# end ModelName;

# Let us understand each part in detail.

# 2.1 Parameters

# Parameters are constants whose values are defined before simulation and do not change during execution.

# Example:

# parameter Real mass = 10;

# parameter Real gravity = 9.81;

# Parameters are typically used for:

- Physical constants

- Design values

- System configuration values

# 2.2 Variables

# Variables represent system states or unknown quantities that change during simulation.

# Example:

# Real velocity;

# Real acceleration;

# Variables are solved by the equation solver during simulation.

# 2.3 Equations

# Equations describe physical relationships between variables.

# Example:

# mass * acceleration = force;

# der(velocity) = acceleration;

# Important:
In Modelica, equations are declarative, not assignment statements.

# This means:

# mass * acceleration = force;

# is not “calculate acceleration”, but rather:

# “mass, acceleration, and force satisfy this relationship.”

# 3. Simple Example – Mass-Spring System

# model MassSpring

# parameter Real m = 5;

# parameter Real k = 100;

# Real x;

# Real v;

# equation

# der(x) = v;

# m*der(v) + k*x = 0;

# end MassSpring;

# Here:

- m and k are parameters

- x and v are variables

- Equations define motion

# The solver determines how variables evolve over time.

# 4. Acausal vs Causal Modeling Concepts

# 4.1 Causal Modeling

# In causal modeling:

- Inputs and outputs are predefined.

- Computation direction is fixed.

# Example (algorithmic style):

# acceleration = force / mass;

# velocity = velocity + acceleration * dt;

# This is procedural and directional.

# Common in:

- MATLAB scripts

- C programming

- Python

# 4.2 Acausal Modeling

# In acausal modeling:

- Only relationships are defined.

- No input/output direction is specified.

- Solver determines computation order.

# Example:

# mass * acceleration = force;

# This equation works regardless of:

- Whether force is known

- Or acceleration is known

# This flexibility is a core strength of Modelica.

# 5. Defense-Based Use Cases and Examples

# Let us consider multiple defense system examples.

# Example 1: Tactical Battlefield Surveillance Drone (TBSD)

# Objective

# Model drone endurance and motor dynamics.

# Modelica Representation

# model DroneEndurance

# parameter Real capacity = 6000;

# parameter Real voltage = 24;

# parameter Real motorPower = 200;

# parameter Real sensorPower = 100;

# Real totalPower;

# Real endurance;

# equation

# totalPower = motorPower + sensorPower;

# endurance = (capacity * voltage) / totalPower;

# end DroneEndurance;

# This model calculates endurance based on system configuration.

# Use Case:

- Validate 4-hour mission requirement.

- Analyze impact of AI processing power increase.

- Study payload impact on battery life.

# Example 2: Missile Flight Dynamics

# Objective

# Model vertical motion of a missile.

# model MissileMotion

# parameter Real mass = 500;

# parameter Real thrust = 20000;

# parameter Real dragCoeff = 50;

# Real velocity;

# Real height;

# equation

# der(height) = velocity;

# mass*der(velocity) = thrust - dragCoeff*velocity;

# end MissileMotion;

# Use Case:

- Evaluate climb rate.

- Study drag impact.

- Analyze thrust reduction scenarios.

# This model uses acausal force balance.

# Example 3: Radar Power Consumption System

# Objective

# Model radar power distribution in a defense vehicle.

# model RadarPowerSystem

# parameter Real radarPower = 1500;

# parameter Real coolingPower = 500;

# parameter Real batteryCapacity = 20000;

# Real totalPower;

# Real operationTime;

# equation

# totalPower = radarPower + coolingPower;

# operationTime = batteryCapacity / totalPower;

# end RadarPowerSystem;

# Use Case:

- Determine radar operation time.

- Optimize cooling design.

- Analyze emergency battery backup duration.

# Example 4: Tank Suspension System

# model TankSuspension

# parameter Real mass = 20000;

# parameter Real stiffness = 500000;

# Real displacement;

# Real velocity;

# equation

# der(displacement) = velocity;

# mass*der(velocity) + stiffness*displacement = 0;

# end TankSuspension;

# Use Case:

- Analyze shock absorption.

- Study terrain impact.

- Improve ride stability.

# 6. Why Acausal Modeling is Important in Defense Systems

# Defense systems involve:

- Multi-domain coupling

- Dynamic behavior

- Physical laws

- Complex integration

# Acausal modeling allows:

- Natural representation of physics

- Reusable components

- Easier system reconfiguration

- Accurate performance validation