ASSIGNMENT 3- Mechanical Modeling Using Graphical Mode in OMEdit

Objective

To design, configure, and simulate mechanical systems using Graphical Mode in OMEdit by assembling predefined mechanical components and analyzing their dynamic response.

SYSTEMS TO BE DEVELOPED

You must develop the following two systems:

- Translational Spring–Mass–Damper System

- Rotational Inertia–Spring–Damper System

Each system must be created as a separate model inside a common package.

PART A – TRANSLATIONAL MECHANICAL SYSTEM

Task 1: Create Package

Create a package named:

MechanicalGraphicalAssignment

All models must be created inside this package.

Task 2: Create Model – TranslationalSystem

Create a model named:

TranslationalSystem

This model will represent a linear spring–mass–damper system subjected to an external force.

Task 3: Add Subcomponents

Inside TranslationalSystem, add the following components and rename them exactly as specified:

• Fixed support → rename as fixedSupport
• Spring → rename as springElement
• Damper → rename as damperElement
• Mass → rename as massBlock
• Force source → rename as appliedForce

Task 4: Assign Parameter Values

Set the following parameter values:

For springElement:
• Spring constant (c) = 1500 N/m

For damperElement:
• Damping coefficient (d) = 80 Ns/m

For massBlock:
• Mass (m) = 10 kg

For appliedForce:
• Force (f) = 200 N (constant force)

Task 5: System Interconnection Requirements

Connect components in the following sequence:

fixedSupport → springElement → damperElement → massBlock

Connect appliedForce directly to massBlock.

Ensure:

• All flanges are properly connected
• No component remains unconnected
• System forms a single continuous mechanical chain

Task 6: Simulation Requirements

Simulate the TranslationalSystem for:

Start Time = 0
Stop Time = 15 seconds

Plot the following variables:

• massBlock.s (Displacement)
• massBlock.v (Velocity)
• massBlock.a (Acceleration)

Task 7: Analysis Requirements

Students must analyze:

• Initial acceleration behavior
• Oscillation pattern
• Damping effect on amplitude reduction
• Final steady-state displacement

Explain why the system behaves as a second-order system.

PART B – ROTATIONAL MECHANICAL SYSTEM

Task 8: Create Model – RotationalSystem

Create another model named:

RotationalSystem

This model represents a torsional spring–damper system with inertia subjected to external torque.

Task 9: Add Subcomponents

Add the following components and rename them exactly as given:

• Fixed rotational support → rename as fixedRot
• Rotational spring → rename as torsionalSpring
• Rotational damper → rename as torsionalDamper
• Inertia → rename as inertiaDisk
• Torque source → rename as appliedTorque

Task 10: Assign Parameter Values

Set the following parameters:

For torsionalSpring:
• Stiffness (c) = 300 Nm/rad

For torsionalDamper:
• Damping coefficient (d) = 0.5 Nms/rad

For inertiaDisk:
• Moment of inertia (J) = 0.05 kg·m²

For appliedTorque:
• Torque (tau) = 15 Nm (constant)

Task 11: System Interconnection Requirements

Connect components in the following sequence:

fixedRot → torsionalSpring → torsionalDamper → inertiaDisk

Connect appliedTorque directly to inertiaDisk.

Ensure:

• All rotational flanges are properly connected
• No incomplete connections exist
• System forms a valid torsional chain