Lab Exercise 5- Building a Complex Electromechanical Model in OMEdit

LAB EXPERIMENT

Building a Complex Electromechanical Model in OMEdit

DC Motor Driven Rotational Load

1. AIM

To build and simulate a multi-domain electromechanical system in OMEdit by combining electrical and mechanical domains and to understand the complete mathematical energy flow.

2. OBJECTIVE

After completing this experiment, students will be able to:

• Combine electrical and mechanical domains
• Configure DC_PermanentMagnet motor correctly
• Understand internal coupling equations
• Perform simulation with proper solver settings
• Interpret electromechanical transient behavior

Real-Life Example

It works like:

• A ceiling fan
• A water pump
• An electric drill
• A robotic motor

When you turn it on:

It starts fast, then stabilizes.

3. SYSTEM OVERVIEW

The system represents a 24V DC motor driving a rotational mechanical load.

Energy Flow:

Electrical Energy → Magnetic Field → Torque → Mechanical Rotation

4. COMPONENTS USED (Correct Library Path)

Electrical Components

Modelica → Electrical → Analog

• Sources → ConstantVoltage
• Basic → Resistor
• Basic → Inductor
• Basic → Ground

Motor Component

Modelica → Electrical → Machines → BasicMachines → DCMachines

• DC_PermanentMagnet

Mechanical Components

Modelica → Mechanics → Rotational → Components

• Inertia
• Damper
• Fixed

5. STEP-BY-STEP PROCEDURE

STEP 1: Launch OMEdit

Open OpenModelica and launch OMEdit graphical interface.

STEP 2: Create a New Model

Click File → New Modelica Class.
Name the model: ElectromechanicalSystem.
Select restriction: model.
Click OK.

STEP 3: Add Electrical Voltage Source

Navigate to Modelica → Electrical → Analog → Sources.
Drag and drop ConstantVoltage into the diagram.

Double-click and set:

V = 24 V

Explanation:
This provides constant DC electrical supply.

STEP 4: Add Electrical Resistor

Navigate to Modelica → Electrical → Analog → Basic.
Drag Resistor into diagram.

Set:

R = 2 Ω

Explanation:
This limits current and represents armature resistance external to motor.

STEP 5: Add Electrical Inductor

From the same Basic package, drag Inductor.

Set:

L = 0.5 H

Explanation:
This represents electrical inductance that resists sudden current change.

STEP 6: Add Ground

Drag Ground from Basic package.

Explanation:
Ground provides electrical reference and completes the circuit.

STEP 7: Add DC Motor

Navigate to Modelica → Electrical → Machines → BasicMachines → DCMachines.

Drag DC_PermanentMagnet into diagram.

STEP 8: Configure DC Motor (General Tab)

Open General tab.

Set:

Jr = 0.01 kg·m²
Js = 0

Explanation:
Jr represents rotor inertia.

STEP 9: Configure DC Motor (Armature Tab)

Open Armature tab.

Set:

Ra = 1 Ω
La = 0.5 H

Explanation:
Ra represents internal armature resistance.
La represents armature inductance.

STEP 10: Configure DC Motor (Nominal Parameters Tab)

Open Nominal parameters tab.

Set:

VaNominal = 24 V
IaNominal = 10 A
wNominal = 1500 rev/min
TaNominal = 25 °C

Explanation:
Torque constant (Kt) and back EMF constant (Ke) are internally derived from these nominal values.

STEP 11: Add Mechanical Inertia

Navigate to Modelica → Mechanics → Rotational → Components.

Drag Inertia into diagram.

Set:

J = 0.02 kg·m²

Explanation:
Represents mechanical load inertia.

STEP 12: Add Mechanical Damper

Drag Damper component.

Set:

d = 0.1 N·m·s

Explanation:
Represents friction and mechanical losses.

STEP 13: Add Fixed Support

Drag Fixed component.

Explanation:
Provides mechanical reference point.

STEP 14: Make Electrical Connections

Connect Voltage.p → Resistor.p
Connect Resistor.n → Inductor.p
Connect Inductor.n → motor.pin_ap
Connect motor.pin_an → Ground.p
Connect Voltage.n → Ground.p

Explanation:
Completes electrical armature circuit.

STEP 15: Make Mechanical Connections

Connect motor.flange → load.flange_a
Connect load.flange_b → damper.flange_a
Connect damper.flange_b → fixed.flange

Explanation:
Transfers torque from motor to load.

6. COMPLETE MATHEMATICAL FLOW

STEP 16: Electrical Equation

V = R_total * i + L_total * (di/dt) + Ke * ω

Where:

R_total = 2 + Ra
L_total = 0.5 + La

Explanation:
Voltage balances resistive drop, inductive drop, and back EMF.

STEP 17: Back EMF Equation

E_back = Ke * ω

Explanation:
Speed generates opposing voltage.

STEP 18: Torque Equation

T = Kt * i

Explanation:
Current produces torque.

STEP 19: Mechanical Equation

(J_total) * (dω/dt) = T − Bω

Where:

J_total = Jr + Load J

Explanation:
Torque accelerates rotor; damper resists motion.

STEP 20: Understand Startup Behavior

At t = 0:

ω = 0
Back EMF = 0
Current = Maximum
Torque = Maximum

As ω increases:

Back EMF increases
Current decreases
Torque decreases

System stabilizes when torque equals damping load.

7. SIMULATION SETTINGS

STEP 21: Open Simulation Setup

Right-click model → Simulation Setup.

STEP 22: Set Simulation Parameters

Start Time = 0
Stop Time = 5

Click Simulate.

8. RESULT VISUALIZATION

STEP 23: Plot Electrical Current

Select:

motor.pin_ap.i

Expected:
High initial current spike.

STEP 24: Plot Angular Speed

Select:

motor.w

Expected:
Gradual increase to steady state.

STEP 25: Plot Electromagnetic Torque

Select:

motor.tauElectrical

Expected:
High at startup, decreases later.

9. ENERGY FLOW ANALYSIS

Electrical Power:

P = V * i

Mechanical Power:

P = T * ω

Losses:

I²R loss
Mechanical damping loss

10. CONCLUSION

This experiment demonstrates true multi-domain modeling in OMEdit where electrical and mechanical physics are coupled automatically and solved simultaneously.