Lab Exercise 7 - Hierarchical & Modular Modeling of Mechanical System (Text View Method)

1. AIM

To design a hierarchical and modular mechanical system in OpenModelica using Text View by creating reusable mechanical subsystems and integrating them into a top-level model.

2. OBJECTIVES

After completing this lab, students will be able to:

Create reusable mechanical Modelica models
Build hierarchical mechanical structures
Connect mechanical subsystems properly
Simulate a modular mechanical system
Understand advantages of modular modeling

3. SOFTWARE REQUIREMENT

OpenModelica (OMEdit)

4. SYSTEM TO BE DEVELOPED

A Modular Mechanical System consisting of:

• Force Subsystem
• Mass Subsystem
• Spring-Damper Subsystem
• Top-Level Integrated Mechanical Model

The system represents a translational second-order mechanical system.

5. STEP-BY-STEP PROCEDURE (TEXT VIEW METHOD)

STEP 1: CREATE A NEW PACKAGE

Open OMEdit

Click File → New Modelica Class

Name the class: ModularMechanicalSystem

Select restriction: package

Click OK

Switch to Text View

Leave the package empty initially.

STEP 2: CREATE FORCE SUBSYSTEM

Right-click the package → New Modelica Class

Name: ForceModule

Restriction: model

Switch to Text View

Paste the following code:

model ForceModule

Modelica.Mechanics.Translational.Sources.ConstantForce force(f=150);

Modelica.Mechanics.Translational.Interfaces.Flange_b outputFlange;

equation

connect(force.flange, outputFlange);

end ForceModule;

This subsystem provides a constant force of 150 N and acts as a reusable force module.

STEP 3: CREATE MASS SUBSYSTEM

Right-click package → New Modelica Class

Name: MassModule

Restriction: model

Switch to Text View

Paste the following code:

model MassModule

Modelica.Mechanics.Translational.Components.Mass mass(m=5);

Modelica.Mechanics.Translational.Interfaces.Flange_a inputFlange;

Modelica.Mechanics.Translational.Interfaces.Flange_b outputFlange;

equation

connect(inputFlange, mass.flange_a);

connect(outputFlange, mass.flange_b);

end MassModule;

This subsystem represents a mass of 5 kg with input and output mechanical connectors.

STEP 4: CREATE SPRING-DAMPER SUBSYSTEM

Right-click package → New Modelica Class

Name: SpringDamperModule

Restriction: model

Switch to Text View

Paste the following code:

model SpringDamperModule

Modelica.Mechanics.Translational.Components.Spring spring(c=1000);

Modelica.Mechanics.Translational.Components.Damper damper(d=50);

Modelica.Mechanics.Translational.Components.Fixed fixed;

Modelica.Mechanics.Translational.Interfaces.Flange_a inputFlange;

equation

connect(inputFlange, spring.flange_a);

connect(spring.flange_b, damper.flange_a);

connect(damper.flange_b, fixed.flange);

end SpringDamperModule;

This subsystem represents a spring (1000 N/m), damper (50 Ns/m), and fixed support.

STEP 5: CREATE TOP-LEVEL MECHANICAL SYSTEM

Right-click package → New Modelica Class

Name: MechanicalDriveSystem

Restriction: model

Switch to Text View

Paste the following code:

model MechanicalDriveSystem

ForceModule forceModule;

MassModule massModule;

SpringDamperModule springDamperModule;

equation

connect(forceModule.outputFlange, massModule.inputFlange);

connect(massModule.outputFlange, springDamperModule.inputFlange);

end MechanicalDriveSystem;

This top-level model connects Force → Mass → Spring-Damper in a hierarchical manner.

STEP 6: VERIFY PACKAGE STRUCTURE

Your final structure must appear as:

STEP 7: SIMULATION PROCEDURE

Right-click MechanicalDriveSystem

Click Simulation Setup

Set:

Start Time = 0
Stop Time = 10

Click Simulate

STEP 8: RESULTS TO PLOT

After simulation, plot:

massModule.mass.s (Displacement)
massModule.mass.v (Velocity)
massModule.mass.a (Acceleration)