Lab Exercise 8 - Extending Mechanical Core Library Component in OpenModelica

AIM

To extend a core translational mechanical component from the Modelica Standard Library, build a complete mass–spring–damper system using the extended component, compute energy and damping losses, and simulate system behavior in OpenModelica.

- OBJECTIVES

After completing this lab, students will be able to:

• Extend a core Modelica library component using inheritance
• Add custom physical calculations
• Build a modular mechanical system
• Compute kinetic and potential energy
• Analyze damping power loss
• Perform simulation and interpret system response

- THEORY OVERVIEW

A translational mechanical system follows Newton’s Second Law:

m d²s/dt² + c ds/dt + k s = F

Where:

m = mass
c = damping coefficient
k = spring stiffness
s = displacement
F = applied force

Energy relationships:

Kinetic Energy = ½ m v²
Potential Energy = ½ k s²
Damping Power Loss = c v²

We will extend the core library component:

This demonstrates professional modeling practice because we do NOT modify library code — we extend it safely.

PART A – EXTENDING A CORE LIBRARY COMPONENT

MAJOR STEP 1 – CREATE A NEW PACKAGE

Step 1.1 Open OMEdit

Step 1.2 Click File → New Modelica Class

Name: MechanicalHierarchicalLab
Restriction: package

Click OK.

This package will contain all models for this experiment.

MAJOR STEP 2 – EXTEND THE CORE MASS COMPONENT

We now extend the Mass model from the Modelica Standard Library.

Create a new model:

Name: ExtendedMass
Restriction: model

Insert the following code:

model ExtendedMass

extends Modelica.Mechanics.Translational.Components.Mass;

Real kineticEnergy "Kinetic Energy";

Real momentum "Linear Momentum";

equation

kineticEnergy = 0.5 * m * v^2;

momentum = m * v;

end ExtendedMass;

DETAILED EXPLANATION OF THE CODE

- extends Modelica.Mechanics.Translational.Components.Mass;

This line means:

• We inherit all equations and connectors of the standard Mass component
• The mass already contains:

- Displacement (s)

- Velocity (v)

- Acceleration

- Force balance equations

- Mechanical flanges

We are not rewriting physics.
We are reusing the existing core component safely.

- Real kineticEnergy;

This declares a new variable to calculate the kinetic energy of the mass.

- Real momentum;

This declares a variable to calculate linear momentum.

- kineticEnergy = 0.5 * m * v^2;

This calculates:

½ m v²

Where:
m = mass parameter
v = velocity of mass

So during simulation, this continuously computes energy based on motion.

- momentum = m * v;

This computes linear momentum of the moving mass.

What this extended model actually does:

It behaves exactly like a normal Mass component,
but now additionally calculates:

• Kinetic energy
• Momentum

This makes it reusable in advanced systems.

PART B – BUILDING THE COMPLETE MECHANICAL SYSTEM

MAJOR STEP 3 – CREATE COMPLETE MECHANICAL DRIVE SYSTEM

Create new model:

Name: MechanicalDriveSystem
Restriction: model

Insert the following code:

model MechanicalDriveSystem

Modelica.Mechanics.Translational.Sources.ConstantForce force(F=15);

ExtendedMass massBlock(m=3);

Modelica.Mechanics.Translational.Components.Spring springElement(c=1500);

Modelica.Mechanics.Translational.Components.Damper damperElement(d=8);

Modelica.Mechanics.Translational.Components.Fixed fixedSupport;

Real potentialEnergy;

Real dampingPower;

Real totalEnergy;

equation

connect(force.flange, massBlock.flange_a);

connect(massBlock.flange_b, springElement.flange_a);

connect(springElement.flange_b, damperElement.flange_a);

connect(damperElement.flange_b, fixedSupport.flange);

potentialEnergy = 0.5 * springElement.c * massBlock.s^2;

dampingPower = damperElement.d * massBlock.v^2;

totalEnergy = massBlock.kineticEnergy + potentialEnergy;

end MechanicalDriveSystem;

DETAILED EXPLANATION OF SYSTEM CODE

- ConstantForce force(F=15);

Applies a constant 15 N force to the system.
This is the input driving force.

- ExtendedMass massBlock(m=3);

A 3 kg mass that:

• Moves under applied force
• Computes kinetic energy
• Computes momentum

- Spring springElement(c=1500);

Spring stiffness = 1500 N/m

This stores mechanical energy when stretched.

- Damper damperElement(d=8);

Damping coefficient = 8 Ns/m

This dissipates mechanical energy as heat.

- Fixed fixedSupport;

Provides a fixed mechanical reference point.

CONNECT STATEMENTS EXPLAINED

connect(force.flange, massBlock.flange_a);

Force acts on the mass.

connect(massBlock.flange_b, springElement.flange_a);

Mass connected to spring.

connect(springElement.flange_b, damperElement.flange_a);

Spring connected to damper.

connect(damperElement.flange_b, fixedSupport.flange);

Damper connected to ground.

This forms:

Force → Mass → Spring → Damper → Fixed Support

A classical mass–spring–damper system.

ENERGY EQUATIONS EXPLAINED

potentialEnergy = 0.5 * k * s²

This calculates spring stored energy.

dampingPower = d * v²

This calculates power dissipated by damper.

totalEnergy = kinetic + potential

This tracks system mechanical energy.

WHAT THE COMPLETE SYSTEM IS ACTUALLY DOING

- Force pushes the mass.

- Mass accelerates.

- Spring resists motion and stores energy.

- Damper removes energy from system.

- Motion gradually stabilizes.

You observe oscillation that decays over time due to damping.

PART C – SIMULATION

MAJOR STEP 4 – SIMULATION SETTINGS

Start Time = 0
Stop Time = 10

Simulate the model.

Plot the following:

massBlock.s (Displacement)
massBlock.v (Velocity)
massBlock.kineticEnergy
potentialEnergy
totalEnergy
dampingPower