Lab Exercise 6 - Hierarchical & Modular Modeling in OMEdit

Creating Reusable Components and Structuring Models for Scalability

1. AIM

To design a hierarchical and modular Modelica system in OMEdit by creating reusable components (subsystems) and integrating them into a scalable top-level system model.

2. OBJECTIVES

After completing this lab, students will be able to:

• Create reusable Modelica components
• Build hierarchical system structures
• Organize models into subsystems
• Improve model scalability and readability
• Understand benefits of modular modeling

3. INTRODUCTION TO HIERARCHICAL MODELING

Hierarchical modeling means:

Building small components first and then combining them into larger systems.

Instead of writing everything in one large model, we:

- Create small reusable blocks

- Test them individually

- Assemble them into a main system

This is called modular design.

4. SYSTEM TO BE DEVELOPED

We will build a:

Modular DC Motor Drive System

The system will be structured into:

- Power Supply Subsystem

- Motor Subsystem

- Mechanical Load Subsystem

- Top-Level System Model

Each subsystem will be reusable.

5. STEP-BY-STEP PROCEDURE

STEP 1: Create a New Package

Click File → New Modelica Class

Name it:

ModularDriveSystem

Select restriction:

package

Click OK.

Explanation:
A package helps organize related models.

STEP 2: Create Power Supply Subsystem

Right-click the package → New Modelica Class

Name:

PowerSupply

Restriction:

model

Click OK.

STEP 3: Add Electrical Components in PowerSupply

Add:

Modelica → Electrical → Analog → Sources → ConstantVoltage
Modelica → Electrical → Analog → Basic → Ground

Set:

V = 24 V

Add electrical connectors:

Modelica → Electrical → Analog → Interfaces → PositivePin
Modelica → Electrical → Analog → Interfaces → NegativePin

Rename them:

p

n

Connect:

Voltage.p → p
Voltage.n → Ground
Ground → n

Explanation:
This subsystem now acts as a reusable DC power module.

STEP 4: Create Motor Subsystem

Right-click package → New Modelica Class

Name:

MotorModule

Restriction:

model

STEP 5: Add Motor and Electrical Interface

Add:

DC_PermanentMagnet motor

Add connectors:

PositivePin
NegativePin

Connect:

p → motor.pin_ap
n → motor.pin_an

Configure motor parameters (as previously learned):

General:
Jr = 0.01

Armature:
Ra = 1
La = 0.5

Nominal:
VaNominal = 24
IaNominal = 10
wNominal = 1500

STEP 6: Add Mechanical Interface to MotorModule

Add mechanical connector:

Modelica → Mechanics → Rotational → Interfaces → Flange_b

Rename as:

shaft

Connect:

motor.flange → shaft

Explanation:
MotorModule now has electrical input and mechanical output.

STEP 7: Create Load Subsystem

Right-click package → New Modelica Class

Name:

LoadModule

Restriction:

model

STEP 8: Add Mechanical Components in LoadModule

Add:

Inertia
Damper
Fixed

Set:

J = 0.02
d = 0.1

Add mechanical connector:

Flange_a

Rename as: inputShaft

Connect:

inputShaft → inertia
inertia → damper
damper → fixed

Explanation:
This subsystem represents mechanical load.

STEP 9: Create Top-Level System

Right-click package → New Modelica Class

Name:

DriveSystem

Restriction:

model

STEP 10: Instantiate Subsystems

Drag into diagram:

PowerSupply
MotorModule
LoadModule

STEP 11: Connect Subsystems

Electrical connections:

PowerSupply.p → MotorModule.p
PowerSupply.n → MotorModule.n

Mechanical connection:

MotorModule.shaft → LoadModule.inputShaft

Now the complete system is built hierarchically.

6. HIERARCHICAL STRUCTURE

Each module can be reused in another project.

7. WHY MODULAR MODELING IS IMPORTANT

ADVANTAGE 1: Reusability

MotorModule can be reused in:

• Robotics
• Electric vehicle
• Industrial drive

ADVANTAGE 2: Scalability

You can replace:

LoadModule with a gearbox
PowerSupply with battery model
MotorModule with another motor

Without changing entire system.

ADVANTAGE 3: Maintainability

Errors are easier to locate.

Each subsystem can be tested independently.

ADVANTAGE 4: Readability

Large systems become clean and structured.

8. SIMULATION SETTINGS

Right-click DriveSystem → Simulation Setup

Start Time = 0
Stop Time = 5

Click Simulate.

9. RESULTS TO OBSERVE

Plot:

MotorModule.motor.w
MotorModule.motor.pin_ap.i

You will observe:

• Current spike at startup
• Speed rising gradually
• Steady-state operation