ASSIGNMENT 1 (Solution )- Modeling a Simple Physics Equation in OMEdit

FreeFallModel – Solution (OMEdit / Modelica Code)

1. Complete Modelica Code

Create a new model in OMEdit named:

FreeFallModel

Paste the following code:

model FreeFallModel

parameter Real g = -9.81;        // gravitational acceleration (m/s^2)

parameter Real m = 1;            // mass of object (kg)

parameter Real h0 = 100;         // initial height (m)

Real h(start = h0);              // height (m)

Real v(start = 0);               // velocity (m/s)

Real a;                          // acceleration (m/s^2)

equation

a = g;                           // constant acceleration due to gravity

der(v) = a;                      // acceleration is derivative of velocity

der(h) = v;                      // velocity is derivative of height

end FreeFallModel;

2. Explanation of Each Section

Parameters

g = -9.81
Represents gravity acting downward.

m = 1
Mass of object (not used in equation because mass cancels in free fall).

h0 = 100
Initial height in meters.

Variables

h
Height of object (changes over time).

v
Velocity of object.

a
Acceleration.

Equations

a = g
Acceleration is constant.

der(v) = a
Velocity changes due to acceleration.

der(h) = v
Height changes according to velocity.

These are declarative equations (not step-by-step instructions).

3. Simulation Instructions

In OMEdit:

- Click Check Model

- Click Simulate

- Set simulation time:

Start time = 0
Stop time = 5 seconds

4. Plot the Following Variables

Plot:

h
v
a

5. Expected Results

Acceleration

Constant at:

-9.81 m/s²

Graph is a straight horizontal line.

Velocity

Linearly decreasing over time.

Because:

v = g × time

Graph is straight line with negative slope.

Height

Parabolic curve decreasing over time.

Because:

h = h0 + 0.5 × g × t²

Graph is a downward-opening parabola.

6. Time to Hit Ground

Object hits ground when:

h = 0

From physics:

t = sqrt(2h0 / 9.81)

For h0 = 100 m:

t ≈ 4.51 seconds

You should observe height reaching zero near this time.

7. Parameter Modification Results

Case 1: Increase Initial Height

If h0 = 200:

• Acceleration remains same
• Velocity profile same slope
• Impact time increases

Because gravity does not change.

Case 2: Change Mass

If m = 10 or m = 100:

• Motion remains identical
• Acceleration unchanged

Reason:

In free fall:

m * a = m * g
Mass cancels → a = g