Lab Exercise 9 - SI Units for Patriot Missile Launcher Rotation System (Simplified Academic Model)

This is a famous surface-to-air missile defence system.
We will NOT model weapons — only the launcher rotation mechanism, which is a safe electromechanical academic system.

OBJECTIVE OF THIS LAB

Simulate a missile launcher azimuth rotation system using:

• DC Voltage Source
• Resistor
• Inductor
• DC Motor (simplified equations)
• Rotational Inertia
• Damper

We will write everything in text editor mode (no graphical view).

PHYSICAL IDEA OF THE SYSTEM

The launcher rotates left or right to aim toward a target.

Power → Motor → Shaft → Launcher Platform

The motor generates torque.
The platform has inertia.
Friction resists motion.

MATHEMATICAL MODEL

Electrical side:

V = R i + L di/dt + k ω

Mechanical side:

J dω/dt = k i − b ω

Where:

V = applied voltage
i = current
ω = angular velocity
J = inertia
b = damping
k = motor constant

STEP 1: CREATE A NEW MODEL

Open OMEdit
File → New Modelica Class
Name: PatriotLauncherSystem

Type: Model

STEP 2: COMPLETE MODEL CODE

Paste the following complete code:

model PatriotLauncherSystem

import Modelica.SIunits;

parameter SIunits.Resistance R = 2 "Armature resistance (Ohm)";

parameter SIunits.Inductance L = 0.5 "Armature inductance (H)";

parameter SIunits.Inertia J = 5 "Launcher inertia (kg.m2)";

parameter Real k = 0.8 "Motor constant (Nm/A)";

parameter Real b = 1 "Damping coefficient (Nms)";

SIunits.Current i(start=0);

SIunits.AngularVelocity w(start=0);

SIunits.Angle theta(start=0);

SIunits.Voltage V;

equation

// Input voltage (step command to rotate launcher)

V = if time < 1 then 0 else 24;

// Electrical equation

L*der(i) + R*i + k*w = V;

// Mechanical equation

J*der(w) = k*i - b*w;

// Angular position

der(theta) = w;

end PatriotLauncherSystem;

STEP 3: SIMULATION SETTINGS

Simulation → Setup

Start Time = 0
Stop Time = 5
Click Simulate.

STEP 4: PLOT RESULTS

Plot:

• theta (angular position)
• w (angular velocity)
• i (current)